# Structure India Trading (P) Ltd.

(A DIV OF Structure India, An ISO 9001:2000 Co.)

Head Off : # 2, Hasanpur Main Road, 52, I.P.Extn, New Delhi-110092. INDIA.

Works : 342/10, Pandav Nagar, B.S.Road Indl Area, Ghaziabad-201010(U.P.)

OFF: 011-22234147/32905880   Cell : +91-9868525835/36

www.structureindia.net                www.ashelectrode.com

REF: Ref: SITPL/QTN/JJBH /NPL/05-2010	DT: .29.05.10

TO,

----------------------

----------------------

SUB: YOUR REQUIREMENT FOR DRINKING WATER PLANT

Dear Sir,

This  refers  to  your  E-mail  with  our  Mr. Prashant Saxena  regarding  your subject requirement  are indeed appreciated.  We thank you very much for sparing your valued time for the discussion.

In this regard, we are submitting herewith our offer for following capacities :

CAPACITY: 5000 LITER / HR. . . . . 1,00,000 LTR /DAY

The detailed offer is as per attached INDEX and ANNEXURES.

We hope that you will find the above in line with your requirement.

Kindly contact us for any further information / clarification require in this matter. We shall be glad to furnish the same.

Thanking you and anticipating a favorable reply. Yours faithfully,

For Structure India Trading (P) Ltd.

(Nivedita )

PROJECT.  CO-ORDINATOR

PACKAGED DRINKING WATER

PLANT

1

| I N D E X |  |  |  |
| --- | --- | --- | --- |
| CLIENT : MR. PROJECT : WATER TREATMENT PLANT |  |  |  |
| CAPACITY : 5 000 LITERS PER HOUR |  |  |  |
| ANNEXURE | DESCRIPTION | PAGE NO. |  |
| WATER PURIFICATION SYSTEM |  |  |  |
| ANNX – I | DESIGN BASIS | Pg.04 |  |
| ANNX – II | OPERATING DATA OF R.O. PLANT | Pg.05 |  |
| ANNX – III | TREATMENT SCHEME | Pg.06 |  |
| ANNX – IV | TECHNICAL DATA OF PLANT | Pg.07 |  |
| ANNX – V | SCHEDULE OF EXCLUSION | Pg.22 |  |
| PACKING M/C |  |  |  |
| • BOTTLING PLANT |  |  |  |
| ANNX – VI | TREATMENT SCHEME OF BOTTLING PLANT | Pg.23 |  |
| ANNX – VII | TECHNICAL DATA OF BOTTLING PLANT | Pg.24 |  |
| ANNX – VIII | TECH.SPECIFICATION & SALIENT FEATURE | Pg.25 |  |
| ANNX – IX | SCHEDULE OF EXCLUSION | Pg.26 |  |
| • POUCH PACKING M/C |  |  |  |
| ANNX – X | TREATMENT SCHEME OF POUCH PACKING M/C. | Pg.27 |  |
| ANNX – XI | TECHNICAL DATA OF POUCH PACKING M/C. | Pg.28 |  |
| ANNX – XII | SCHEDULE OF EXCLUSION | Pg.29 |  |
| ISI EQUIPMENTS (If Required) |  |  |  |
| ANNX – XIII | LABORATORY EQUIPMENTS | Pg.30 |  |
| ANNX –XIV | LIST OF CHEMICALS | Pg.31 |  |
| ANNX – XV | LIST OF GLASSWARE ITEMS | Pg.35 |  |
| ANNX – XVI | SCHEDULE OF EXCLUSION | Pg.36 |  |
| PRICE, TERMS & CONDITIONS |  |  |  |
| ANNX – XVII | TOTAL PROIECT FOR WATER TREATMENT PLANT | Pg.37 |  |
| ANNX – XVIII | TERMS & CONDITIONS | Pg.38 |  |
| WHY DRINKING WATER PROJECT |  |  |  |
| ANNX—XIX | • IIECI-HELP LINE • CAPITAL COST – INTEREST COST • COST OF PRODUCTION • PROFITIBILITY CHART | Pg.39 |  |
| ANNX – XX | SPECIAL FEATURES OF IIECI | Pg.44 |  |
| ANNX – XXI | CLIENT LIST FOR DRINKING WATER PLANT | Pg.45 |  |

A N N E X U R E – I

DESIGN	BASIS OF THE PLANT

BASIS OF DESIGN :-

The following raw water analysis is considered as basis of design the

Plant.

A. RAW WATER CHARACTERISTICS

7.0 – 7.5 Mg/Ltr

2000 mg/Ltr.

| 200 mg/ Ltr. pH |  | ..... |
| --- | --- | --- |
| Total | dissolved solids | .... |
| Total | hardness as CaCo3 | ..... |

| M. alkalinity as CaCo3 | ..... | 200 | mg/ | Ltr. |
| --- | --- | --- | --- | --- |
| P. alkalinity as CaCo3 | ..... | NIL | mg/ | Ltr. |

NB 	: WE REQUEST YOU TO KINDLY SEND US YOUR ANALYSIS REPORT, INCLUDING

(T.D.S, 	PH, 	TOTAL 	HARDNESS, 	SILICA, 	IRON, 	COD, 	BOD, 	SULPHATE, CHLORIDE, etc)

B. TREATED WATER QUALITY FROM R.O.PLANT:

0 ppm

7.0 - 7.5

| <100-150 Mg/Ltr. Total | Hardness |  | ...... |
| --- | --- | --- | --- |
| PH |  |  | ...... |
| Total | Dissolved | solids | ...... |

The above quality of R.O. Product water is achieved, subject to following conditions :-

01. The feed water quality is not worse than specified.

02. The feed water limiting condition mentioned below are strictly maintained.

03. The operation & maintenance of the entire system is carried out strictly as per our operation & maintenance manual.

04. Pure Cleaning Chemicals, Original spare & consumables specified by us are used in plant.

A N N E X U R E – II

OPERATING 	DATA OF PLANT

CAPACITY: 5000 LTRS PER HR. .. (1,00,000 LTR/DAY)

| * | No of stream offered | . | . | . | One |
| --- | --- | --- | --- | --- | --- |
| * | Mode of Plant Operation | . | . | . | Manual |
| * | Feed Water Flow Rate | . | . | . | 8.0 M3/Hr. |
| * | Product Flow Rate | . | . | . | 5 M3/Hr. |
| * | Average recovery | . | . | . | 60% |
| * | Feed Water Pressure | . | . | . | 2-12 Kg/Cm2 |
| * | Working hours / day | . | . | . | 20 Hrs. |
| * | Product Water produced/day | . | . | . | 10 M3/Day |

R.O. FEED WATER LIMITING CONDITIONS :

| * | Turbidity |  | . | . | . | <1 NTU S.D.I. | <5 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| * | Heavy Metal |  | . | . | . | Nil |  |
| * | Organics and | Bacteria | . | . | . | Nil |  |

* Oil 	& Grease 	. . . Nil

| * | Residual chlorine | . | . | . | Nil |
| --- | --- | --- | --- | --- | --- |
| * | Temperature Deg. C. (Max.) | . | . | . | < 40 |
| * | Odor | . | . | . | Odorless |

A N N E X U R E – III

TREATMENT 	SCHEME OF PLANT

RAW 	WATER 	TANK- CLIENT SCOPE

RAW WATER PUMP – CLIENT SCOPE E.M.F.TECHNOLOGY-OPTIONAL DUAL MEDIA FILTER (DMF) ANTHRACITE FILTER (AF) SOFTENER ( SF )

MICRON CARTRIEDGE FILTER

HIGH PRESSURE PUMP – GROUNDFOS/DP/KIRLOSKAR REVERSE OSMOSIS SYSTEMS

R.O. WATER STORAGE TANK-CLIENT SCOPE

SPRING TECHNOLOGY-OPTIONAL

MICRON CARTRIDGE FILTER – POST TREATMENT GAC FILTER-OPTIONAL

U.V. SYSTEM

OZONE SYSTEMS-OPTIONAL OZONE PUMP-OPTIONAL OZONE TANK-OPTIONAL END / USE

A N N E X U R E – IV

TECHNICAL DATA OF THE PLANT

01. 	RAW WATER SUPPLY PUMP (CS)

One No. 	S.S./C.I. Construction are 	required 	for 	feeding 	Raw 	water

from 	underground 	/ 	 overhead 	raw 	water 	storage 	tank 	to 	Multi 	Media Filter 		to 	Softener 		to(Micron 		Cartridge 	 Filter) 		with flow 	rate with pressure. 	The 	pump 		is 	fitted 	with 		suction 	/ 	discharge 	 pipe 	 work 	and isolation valves.

Quantity Offered 	: 	One No.

Type 	: 	Centrifugal/ Monoblock

Operating Flow Rate 	: 	80.0 M3 / Hr. Operating Pressure 	: 	2 kg/cm2. Material of Construction 	: 	Cast Iron/SS

Type of Drive 	: 	III Phase or I Phase

Make 	: 	Crompton / Kirloskar / Eqv .

02. 	EMF TECHNOLOGY (EMF)-OPTIONAL

Process:

Indian Ion Exchange introduces EMF a latest state of art water treatment technology based on Electro Negativity and Electro Motive Force. It’s a online treatment system non- chemical, without power & Resins.

Function:

EMF system is processed of sixteen Precious and semi Precious metal with thousands of ventures and thousands of Cathode and Anodes.

Water Route:

RAW WATER PUMP    EMF TECHNOLOGY       DUAL MEDIA FILTER Quantity Offered 	: 	One No.

Type 	: 	Down Flow

Operating Flow rate 	: 	8.0 M3

Operating Pressure 	: 	2Kg/cm2. M.O.C. of Shell 	: 	S.S.R.L.

Process:

This is a Stainless Steel inside Rubber Lining (SSRL) pressure vessel, with 	internal 	distribution 	& 	collection 	systems. 	The 	frontal and interconnecting pipes and requisite 	ITALIC valves of BRASS construction along with one lot of sand, silex & pebbles. The necessary instruments like inlet / outlet, pressure gauges are provided with the system.

Function:

The physical impurities of raw water i.e. suspended solids, dirt, dust, &

all visible particles are removed in 	DUAL MEDIA FILTER up to 50 micron.

Water Route :-

EMF TECHNOLOGY    DUAL MEDIA FILER      ANTHRACITE FILTER

| Type |  |  |  |  |  | : | DOWNFLOW |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Quantity |  | Offered |  |  |  | : | ONE |  |  |  |  |  |  |  |
| Material |  | of Construction |  |  |  | : | SSRL 316 L-Rubberling |  |  |  |  |  |  |  |
| Operating Pressure |  |  |  |  | : |  | 0.5 kg-Min-2kg-Normal-3kg-Max |  |  |  |  |  |  |  |
| Backwash Pressure |  |  |  |  | : |  | 0.5 kg Min-2kg-Normal-3kg-Max |  |  |  |  |  |  |  |
| Operating Flow Rate |  |  |  |  | : |  | 8m3/hr |  |  |  |  |  |  |  |
| Type of Media |  |  |  |  | : |  | Dual, Sand, Pebbales |  |  |  |  |  |  |  |
| Supporting Media |  |  |  |  | : |  | Gravels & Silex |  |  |  |  |  |  |  |
| Distribution System |  |  |  |  | : |  | One |  |  |  |  |  |  |  |
| Collecting System |  |  |  |  | : |  | One |  |  |  |  |  |  |  |
| SHELL: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Dimension |  |  |  |  | : |  | Diameter - 450 mm |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Height - 1700 mm |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | Thickness –2 mm |  |  |  |  |  |  |  |
| R.L. Thickness |  |  |  |  | : |  | 5 mm |  |  |  |  |  |  |  |
| PIPE : |  |  |  |  |  |  | Service Backwash Regn |  |  |  |  |  |  |  |
| Type |  |  | : |  |  |  | Sch.10 | Sch.20 |  | - |  |  |  |  |
| MOC |  |  | : |  |  |  | SS-316 | AP |  |  |  |  |  |  |
| Size | (Dia) |  |  | : |  |  | 32 mm |  |  |  |  |  |  |  |
| Qty. |  |  |  | : |  |  | One lot |  | One lot |  | One lot |  |  |  |

| VALVE |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type |  | : | BALL VALVE |  |  |
| MOC |  | : | SS OUTER | COVER BRASS |  |
| Size |  | : | 32 mm |  |  |
| Make | : |  | Italy - Imported |  |  |
| Qty | : |  | 5 Nos. |  |  |

INSTRUMENT : (Pressure Gauge)

| Qty | : | One |
| --- | --- | --- |
| MOC | : | SS |
| Range | : | 0-7 kg. |
| Connection | : | 1/4” |

Process:

This is a SSRL pressure vessel, with internal distribution, collection, frontal and interconnecting  pipes and requisite ITALIC valves of BRASS construction along with one lot of Charcol & Anthracite & supporting media Pebbles & Gravels. The necessary instruments like inlet / outlet, pressure gauges are provided with the system.

Function:

The physical impurity of raw water i.e. suspended solids, dirt, dust, & Color & Odor are removed in 	ANTHRACITE FILTER.

Water Route:

DUAL MEDIA FILTER      ANTHRACITE FILTER      SOFTENER

| Type |  |  | : | DOWNFLOW |  |
| --- | --- | --- | --- | --- | --- |
| Quantity | Offered |  | : | ONE |  |
| Material | of Construction |  | : | SSRL 316 L-Rubberling |  |
| Operating Pressure |  | : |  | 0.5 kg-Min-2kg-Normal-3kg-Max |  |
| Backwash Pressure |  | : |  | 0.5 kg Min-2kg-Normal-3kg-Max |  |
| Operating Flow Rate |  | : |  | 8m3/hr |  |
| Type of Media |  | : |  | Anthracite, Dual, Sand, Pebbales |  |

| Silex Supporting Media | : | Gravels & |
| --- | --- | --- |
| Distribution System | : | One |
| Collecting System | : | One |
| SHELL: |  |  |
| Dimension | : | Diameter |
|  |  | Height |
|  |  | Thickness |
| R.L. Thickness | : | 5 mm |

- 450 mm

- 1700 mm

– 2 mm

PIPE : 	Service 	Backwash 	Regn

| Type |  | : |  | Sch.10 | Sch.20 |  | - |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MOC |  | : |  | SS-316 | AP |  |  |  |  |
| Size | (Dia) |  | : | 32 mm |  |  |  |  |  |
| Qty. |  |  | : | One lot |  | One lot |  | One lot |  |

| VALVE |  |  |  |
| --- | --- | --- | --- |
| Type | : | BALL VALVE |  |
| MOC | : | SS OUTER | COVER BRASS |
| Size | : | 32 mm |  |

Make 	: 	Italy - Imported

Qty 	: 	5 Nos.

INSTRUMENT : (Pressure Gauge)

Qty 	: 	One

MOC 	: 	SS

Range 	: 	0-7 kg. Connection 	: 	1/4”

Process:-

This is a SSRL pressure vessel, with internal distribution, collection, frontal & interconnecting pipes and requisite valves of SS construction along with one lot of RESIN. The necessary instruments like Inlet/outlet, pressure gauges are provided with the system.

Function:-

The 	hardness 	of 	Raw 	Water 	(temporary 	& 	permanent) 	are 	removed 	from

SOFTNING SYSTEM.

Water Route:

ANTHRACITE 	FILTER    	SOFTENER 	 MICRON CARTRIDGE FILTER

| Type |  |  |  |  |  |  |  | : | DOWNFLOW |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Quantity | Offered |  |  |  |  |  |  | : | ONE |  |  |  |  |  |  |
| Material | of Construction |  |  |  |  |  |  | : | SSRL 316 L |  |  |  |  |  |  |
| Operating Pressure |  |  |  |  |  | : |  |  | 0.5kg-Min-2kg-Normal-3kg-Max |  |  |  |  |  |  |
| Backwash Pressure |  |  |  |  |  | : |  |  | 0.5kg-Min-2kg-Normal-3kg-Max |  |  |  |  |  |  |
| Operating Flow |  |  | Rate |  |  |  | : |  | 8m3/hr |  |  |  |  |  |  |
| Type of Media |  |  |  |  |  |  | : |  | Resin |  |  |  |  |  |  |
| Qty of Resin |  |  |  |  |  |  | : |  | 200 Ltrs |  |  |  |  |  |  |
| Regn Ratio |  |  |  |  |  |  | : |  | 80 m3/Regn |  |  |  |  |  |  |
| Design basis |  | on |  |  |  |  | : |  | 150 ppm Total Hardness |  |  |  |  |  |  |
| Distribution |  | System |  |  |  |  | : |  | One |  |  |  |  |  |  |
| Collecting System |  |  |  |  | : |  |  |  | One |  |  |  |  |  |  |
| SHELL: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Dimension |  |  |  |  | : |  |  |  | Diameter - 450 mm |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | Height - 1700 mm |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | Thickness – 2 mm |  |  |  |  |  |  |
| R.L. Thickness |  |  |  |  | : |  |  |  | 5 mm |  |  |  |  |  |  |
| PIPE : |  |  |  |  |  |  |  |  | Service Backwash Regn |  |  |  |  |  |  |
| Type |  |  |  | : |  |  |  |  | Sch.10 | Sch.10 |  | - |  |  |  |
| MOC |  |  |  | : |  |  |  |  | SS-316 | AP |  |  |  |  |  |

| Size | (Dia) | : | 32 mm |  |  |
| --- | --- | --- | --- | --- | --- |
| Qty. |  | : | One lot | One lot | One lot |

| VALVE: |  |  |  |
| --- | --- | --- | --- |
| Type | : | BALL VALVE |  |
| MOC | : | SS OUTER | COVER BRASS |
| Size | : | 32 mm |  |

Make 	: 	Italy – Imported / equiv

Qty 	: 	4 + 1 Nos.

INSTRUMENT: (Pressure Gauge)

Qty 	: 	One

MOC 	: 	SS

Range 	: 	0-7 kg. Connection 	: 	1/4”

Process :-

This is a Stainless Steel Inside Rubber Lining (SSRL) pressure vessel, with 	internal 	distribution 	& 	collection 	systems. 	The 	inlet 	& 	outlet connection with Isolation Valves along with 	micron filter cartridge & requisite instrument.

Function:-

The 	micron 	physical 	impurities 	of 	raw 	water 	i.e. 	suspended 	solids, dirt, dust, & all visible particles are removed in MICRON CARTRIDGE FILTER up to micron.

Water Route:-

SOFTENER    	MICRON CARTRIDGE FILTER       HIGH PRESSURE PUMP

| Qty |  |  |  | : |  | One |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage |  |  |  | : |  | 4 STAGE |  |  |  |
| M.O.C. |  |  |  | : |  | SSRL 316L |  |  |  |
| Operating | Pressure |  |  | : |  | 2KG/CM2 |  |  |  |
| Operating | Flow Rate |  |  | : |  | 8M3/HR |  |  |  |
| Type of Media |  |  |  |  | : | Cartridge |  |  |  |
| SHELL: (Each Stage) |  |  |  |  |  |  |  |  |  |
| MOC |  |  |  |  | : | SS316L |  |  |  |
| Dia |  |  |  |  | : | 125 MM |  |  |  |
| Height |  |  |  |  | : | 600 MM |  |  |  |
| Thickness |  |  |  |  | : | 2 MM |  |  |  |
| PIPE: |  |  |  |  |  |  |  |  |  |
| MOC |  | : |  |  |  | SS | 316 |  |  |
| Schedule |  | : |  |  |  | 10 |  |  |  |
| Inlet connection |  | : |  |  |  | 32 | MM |  |  |
| Outlet connection |  | : |  |  |  | 32 | MM |  |  |
| Type of connection |  |  | : |  |  | FLANGED |  |  |  |
| CARTRIDGE |  |  |  |  |  |  |  |  |  |
| Type |  |  | : |  |  | SPUN |  |  |  |
| Qty |  |  | : |  |  | 4 NOS. |  |  |  |
| Rating |  |  | : |  |  | 5 MICRON |  |  |  |
| Dia |  |  | : |  |  | 62.5 MM |  |  |  |
| Length |  |  | : |  |  | 500 MM |  |  |  |
| Make |  |  | : |  |  | OSMONIC/AMITECH/EQUIV. |  |  |  |
| Accuracy |  |  | : |  |  | +5% |  |  |  |

Process:-

One No. of pump with SS / BRASS construction. The Necessary instruments like High & Low pressure switch, pressure gauges & isolation valves are provided for necessary control of this system.

Function:

The high pressure pump boost purified water pressure to 8-12 KG/cm2 &

provided to RO systems with design flow rate.

Water Route:

MICRON CARTRIDGE FILTER      HIGH PRESSURE PUMP        RO SYSTEMS Type 	: 	Centrifugal / Multistage MOC 	: 	SS

Qty 	: 	1 No.

Operating Pressure 	: 	6-16 kg Operating Flow Rate 	: 	8M3/hr Power (k.w) 	: 	3 k.w

Process :

This system consist of a M.S. Skid for mounting of high pressure tubes with spiral wound membrane elements per tube with necessary control valves at feed, product and reject pipes are provided with required instruments

Low 	& High pressure switches are provided for safe operation of High Pressure Pump under 	specified operating condition. Pressure 	gauges is provided for pressure Indication and control of complete R.O. System. A online type of Flow Indicator 	at 	product & brine pipe work are provided for controlling desired flow rate & recovery. For 	monitoring product water quality a online conductivity indicator is 	provided.

Function :

Physical Impurity 	: Removes 100% Phy.Imp. of raw water. Biological Impurity 	 :Removes 100% Bio.Imp of raw water. Chemical Impurity 	:Removes 90-95% TDS of raw water.

Water Route:-

HIGH PRESSURE PUMP      RO SYSTEM        PRODUCT WATER STORAGE TANK 	OR 	END USE.

R.O. Module:

Make 	: 	KOCH/HYDRANAUTICS/G-BETZ

| Operating | Flow | : | 8 m3/HR |
| --- | --- | --- | --- |
| Operating | Pressure | : | 6-16 KG. |
| Recovery |  | : | 65 % |
| Dia |  | : | 101 MM |
| Length |  | : | 40” |
| Nos. |  | : | 20 |

| R.O. Module Housing: |  |  |
| --- | --- | --- |
| M.O.C. | : | SS-316 |
| Dia-OD | : | 212 MM |
| Length | : | 80“ |
| End Connection | : | FLANGED |
| No. of Housing | : | 10 NOS. |

Process :-

This is a HDPE vertical storage tank is required 	for storage of R.O. Product water. Inlet/Outlet PVC/HDPE pipe work with suitable isolation valves, levels Indicator, etc. are provided.

Water Route:-

R.O.SYSTEM 	 	R .O. WATER STORAGE TANK-      SPRING WATER TECHNOLOGY Quantity Offered 	: 	One No.

Material of Construction 	: 	SS

Capacity 	: 	1000 LTR

WHAT IS SPRING WATER:

Spring Water is generally the water falls from Mountain.

Process :-

In nature, when the water fall from Mountain, it passes through 		various stone, metals, etc with velocity & impact thrust. During the time Air the resist 	it 	with 	its 	own 	velocity 	from 	bottom 	to 	top. 	Heavy 	friction results in remaining of gaseous impurity from water. Degasification  & enrichment of Oxygen (O2). Hence Sweetness becomes is part of Spring Water.

Function:-

The same natural technology is accepted by us, 	Here we provide one SS vessels, where, we inlet the water from Top of the systems and Enter the Air thorough Compressor from Bottom side. The aeration process between water and Air in center chamber passes various venturies & metals. The cavity gums water a fine filtration touch & also it is designed in such a friction that the water gets maximum time 		with air for 	artificial aeration. 		When 	the 		(Aeration) 		process 	are 	 working, 	 the 	Gaseous Impurities are separated and removed from 	Top of the Vessels though head Nozzle. The water after Aeration process becomes a Sweeter, free from Physical, 	Chemical, 	Biological 	and 	most 	 importantly 	that 	Gaseous Impurities.

Water Route:-

R .O. WATER STORAGE TANK          SPRING WATER TECHNOLOGY          MICRON CARTRIDGE FILTER

Quantity offered 	: 	One Nos. Type 	: 		Down Flow Operating Flow rate 	: 	8 M^3/Hr. Operating Pressure 	: 	2.0 Kg/cm2. M.O.C. of Shell 	: 	S.S.316 L

Process:

This is a Stainless Steel Inside Rubber Lining (SSRL) pressure vessel, with 	internal 	distribution 	& 	collection 	systems. 	The 	inlet 	& 	outlet connection with Isolation Valves along with micron filter cartridge & requisite instrument.

Function:

The 	micron physical impurities of raw water 	i.e. suspended solids, dirt, dust, & all 	visible particles are removed in 	MICRON CARTRIDGE FILTER up to 	micron.

Water Route:

SPRING WATER TECHNOLOGY    	MICRON CARTRIDGE FILTER       GAC FILTRATION

| Qty |  | : | One |
| --- | --- | --- | --- |
| Stage |  | : | 3 STAGE |
| M.O.C. |  | : | SSRL 316L |
| Operating | Pressure | : | 1KG/CM2 |

| Operating Flow Rate | : | 8M3/HR |
| --- | --- | --- |
|  | : | Cartridge |
| Stage) |  |  |
|  | : | SS316L |

Type of Media

SHELL : (Each

MOC

| Dia | : | 125 | MM |
| --- | --- | --- | --- |
| Height | : | 600 | MM |
| Thickness | : | 1.5 | MM |
| PIPE: |  |  |  |
| MOC | : | SS | 316 |
| Schedule | : | 10 |  |
| Inlet connection | : | 32 | MM |
| Outlet connection | : | 32 | MM |

Type of connection 	: 	FLANGED

| CARTRIDGE | : |  |  |
| --- | --- | --- | --- |
| Type |  | : | SPUN |
| Qty |  | : | 3 NOS. |
| Rating |  | : | 5 MICRON |
| Dia |  | : | 62.5 MM |
| Length |  | : | 500 MM |
| Make |  | : | OSMONIC/AMITECH/EQUIV. |
| Accuracy |  | : | +5% |

Process:

GAC Filtration is important water of purification system of mineral water. 	While 	doing 	reverse 	osmosis 	the 	Total 	Dissolved 	Solid 	are reduced but the ratio of Ca to TDS, Cl to Na, TH to Alk etc., are not changed. 	 Hence the taste traces remain same as Raw water.

Function:

The Zeta potential granule anthracite combination 	will make above ratio changed in its limit. 	Hence, polishing of product water and improvement of good taste while processing through GAC

Water Route:

MICRON CARTRIDGE FILTER       GAC FILTRATION    ULTRA VIOLET SYSTEM Type 	: 	ZETA PLUS

Quantity offered 	: 	One No. Operating Flow rate 	: 	8 M^3/Hr Operating Pressure 	: 	2 Kg/cm2.

Process :-

This system consist of U.V.LAMP in SS body with intenal wiring, safety instruments indication lamps, ON-OFF switch and inlet-outlet of 1/2".

Function:-

The systems is meant for removal of 	all kind of biological impurities from the water before filling.

WATER ROUTE :

GAC FILTER        U.V SYSTEM      OZONE SYSTEM. M.O.C 	: 	SS 316 L

Make 	: 	SANKYO-JAPAN

Operating flow 	: 	85 M3/HR Operating Pressure 	: 	1 KG/CM2

M.O.C of housing 	: 	SS 316

Phase 	: 	I PHASE

Sand Filter

ACF

6 KL Raw water tank

Bore

Well

Softener

5M

HP P

UV

UV

1M

0.5     M

0.2M

KL Process Storage Tank

KL Drinking PET Tank optional

O3

Second RO optional

RO Reject

RO System 5 stage

Cl2 Dosing

SMBS

PHC

Antiscalant

TDS

ORP

TDS

WTP & RO Flow Diagram for  LIBYA Plant

Optional 2.5kl Holding Tank

R O PLANT

14. 	OZONE

1) OZONE SYSTEM: Process:

Ozone is a Tri 	atmospheric form of Oxygen, which unstable gas made from oxygen 	form 	atmospheric 	air 	which 	is 	use 	to 	oxidize 	all 	harmful Bacteria, Virus, Organic Matter, & Bad odor presence in water.

Function:

Hence Ozone treat water becomes total bacteria free & pleasing in taste. Because dissolved oxygen level in water improve the taste of 	water. Hence, Drinking Water becomes Pleasing.

Water Route:

ULTRA VIOLET SYSTEM       OZONE SYSTEM        OZONE PUMP

| Number of | stream offered | : | One |
| --- | --- | --- | --- |
| Operating | flow rate | : | 5 m3/hr |
| Operating | Pressure | : | 6-16 kg/cm2 |
| 2) OZONE | TANK: |  |  |

Process :-

This is a HDPE vertical storage tank is required 	for storage of ozone Product water. Inlet/Outlet PVC/HDPE pipe work with suitable isolation valves, levels Indicator, etc. are provided.

Water Route:-

OZONE SYSTEM      OZONE TANK        OZONE PUMP

| Quantity Material Capacity Offered | : | One No. |
| --- | --- | --- |
| of construction | : | S.S. |
|  | : | 1000 ltr |

3) OZONE PUMP:

Process:

Ozone water transfer pump lifts the water from ozone tank & feed the same to again ozone tank traveling to ozone ejection system.

This pump circulate the ozone gas in ozone tank for making ozone gas more concentrated in ozone tank.

The M.O.C of Pump is SS/Brass/Non- metal for wetted parts. 	The outer body , prefarebly will be of 	SS.

Function:

To transfer the water from ozone system to ozone tank.

| Quantity Offered | : | One No. |
| --- | --- | --- |
| Material of Construction | : | Stainless Steel / Brass |

15. 	R.O. PANEL WITH INSTRUMENTS

|  | 1. | STRUCTURE |  |  |  |  |  | : |  |  | MS Base |  |  |  | with |  | SS | cover |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2. | ELECTRIC PANEL: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | a. | AMP Meter |  |  |  |  | : |  | 1 | No. |  |  |  |  |  |  |
|  |  |  | b. | Volt Meter |  |  |  |  | : |  | 1 | No. |  |  |  |  |  |  |
|  |  |  | c. | Hours Meter |  |  |  |  | : |  | 1 | No |  |  |  |  |  |  |
|  |  |  | d. | Hooter |  |  |  |  | : |  | 1 | NOS |  |  |  |  |  |  |
|  |  |  | e. | M.C.B |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | • Qty. | : |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | • Make | : |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | • Main | : |  |  |  | 1 | No. |  |  |  |  |  |  |
|  |  |  |  |  |  | • RWP | : |  |  |  | 1 | No. |  |  |  |  |  |  |
|  |  |  |  |  |  | • HPP | : |  |  |  | 1 | No. |  |  |  |  |  |  |
|  |  | f. |  |  | Rely |  |  |  | : |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | • Rwp |  |  |  | : |  | 1 | No. |  |  |  |  |  |  |
|  |  |  |  |  | • HPP |  |  |  | : |  | 1 | No. |  |  |  |  |  |  |
|  |  | g. |  |  | Push Button |  |  |  | : |  | 2 | Nos. |  |  |  |  |  |  |
| 3. |  | FLOW |  |  | METERS: |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | • Type |  |  |  | : | BURDEN |  |  |  |  |  |  |  |
|  |  |  |  |  |  | • Qty |  |  |  | : | 2 NOS |  |  |  |  |  |  |  |
|  |  |  |  |  |  | • Make |  |  |  | : | SHREEJI |  |  |  |  |  |  |  |
|  |  |  |  |  |  | • Capacity |  |  |  | : | 300-200 |  |  |  | LPH |  |  |  |
|  |  |  |  |  |  | • Connection |  |  |  | : | 32 MM |  |  |  |  |  |  |  |

•	Inlet connection : 	Type – Threaded 	Size-32mm

•	Outlet connection: Type - Threaded 	Size-32mm

| • Accuracy : • Qty | : | 2 NOS. |
| --- | --- | --- |
| • Make | : | EDOSE |
| • Capacity | : | 1) 0-5 K.G |
| • Connection | : | ¼” |

4. 	PRESSURE SWITCH 	:

2) 15-17 K.G.

5. 	PRESSURE GAUGE:

| • Type |  | : |  |
| --- | --- | --- | --- |
| • Qty | : | 4 NOS |  |
| • Make |  | : | SHREEJI |

•  Range 	: 	0-28 KG

| • M.O.C | : | SS |
| --- | --- | --- |
| • Connection | : | 3/8’’ |

6. 	SAMPLING POINT:

| • No. | : | 2 NOS |
| --- | --- | --- |
| • Connection | : | ½’’ |

•   M.O.C. 	: 	SS ( CROMPLATED)

7. 	TDS METER:

| • Type | : | Handy/Online |
| --- | --- | --- |
| • Make | : | UTECH |
| • Range | : | 0-1999 |
| • M.O.C | : | PP |

A N N E X U R E – V

SCHEDULE	OF 	EXCLUSION

01. 	All civil works, such as plant building foundation of equipments

pipe support, drain trenches, supporting structures & platforms.

02. 	Final grouting of all the equipments.

03. 	Plant area, building illumination, ventilation & earthing.

04. 	Supply of power, water, Dosing chemicals for Commissioning.

05. 	Laboratory testing facilities, instruments and regenerates.

06. 	Lubricant, oil, grease as required after initial operation.

07. 	All pipe work beyond our termination points.

08. 	Supply of Raw, Soft & Treated Water Storage Tanks & Transfer pumps.

09. 	All 	equipments, 	work 	and 	services 	not 	specifically 	mentioned 	as forming part of our offer.

10. 	Final painting of the equipments / plant supplied.

TERMINATION POINTS

01. 	Inlet of MCF Unit. 	}

} 3 mtrs from respective

02. 	Overflow & drain pipe work } units.

}

CLIENT TO PROVIDE THE FOLLOWING FOR PLANT OPERATION

Control Supply : 220 V AC +/- 10, Single phase,50 HZ for instrument control supply.

A N N E X U R E – VI

TREATMENT  SCHEME OF BOTTLING M/C

BOTTLE WACHING UNIT

BOTTLE CONVEYING UNITS BOTTLE FILLING UNIT BOTTLE SEALNG UNIT

BOTTLE LABLE SHRINK MACHINE

BOTTLE CONVEYAING UNIT END USE

A N N E X U R E – VII

TECHNICAL DATA OF BOTTLING M/C

1. 	SEMI AUTO BOTTLE WASHING MACHINE

| MOC | : | 316 SSRL |
| --- | --- | --- |
| Pump | : | SS PUMP MOTOR |

2. 	AUTOMATIC SIX HEAD MINERAL WATER FILLING & CAPING MACHINE

| Diamension No.of goods Bottle Detail : | 3600 X 1000 X | 1500 Appx. |
| --- | --- | --- |
| : | Six |  |
| : | Min Max |  |

| Body Dia | : |  | 25 | mm 75 mm |  |
| --- | --- | --- | --- | --- | --- |
| Neck Dia | : |  | 10 | mm 40 mm |  |
| Height | : |  | 55 | mm 275 mm |  |
| Out Put |  | : | 16-24 Bottle Per0 Minute |  |  |
|  |  |  | (speed depend up on volume) Accuracy of Fill : |  |  |
|  |  |  | +% of the volume to be filled |  |  |
| Electrical |  | : | Main motor 1 h.p./1440 RPM / |  |  |
|  |  |  | III Phase |  |  |
| Compress Air |  | : | 20 CFM at 5 Bar |  |  |
|  |  |  | (provided by buyer) |  |  |
| NOTE: |  |  |  |  |  |

•	Air-compressor is not provided with machine is to be arranged by the buyer.

•	Strong tank to filling inlet pipeline is to be provided by the buyer

3. 	SHRINK TURNEL ( 6” LENGTH CONVEYOR )

MODEL 	: 	SS GMP

TECHNICAL SPECIFICATION :

| Length | of | passage |  | : | 30” |  |
| --- | --- | --- | --- | --- | --- | --- |
| Width | of | passage |  | : | 6” |  |
| Height | of | passage |  | : | 14’’ |  |
| Temperature |  |  | : |  | 70 to 250 |  |
| Material |  |  | : |  | PVC/LDPE |  |

Power 	load 	: 	6 KV

Motor:-

| For conveyor | : | 05 HP/1440RPM/III Phase |
| --- | --- | --- |
| For fan | : | 0.25HP/I Phase |
| SALIENT FEATURE: |  |  |

Easy to operate

Virtually no maintenance

Low Power consumption

Electronic digital temperature controller

High efficiency heaters

Suitable for PVC & LDPE films

Fixed / variable conveyer speed

A N N E X U R E –IX

SCHEDULE OF EXCLUSION

•	AIR COMPRESSOR OF 3 H.P. AIR FLOW OF 12 C.F.M.

•	POWER CONNECTION

a. 	SHRUNK TUNEL-6 H.P. b. 	ROUND TABLE-1 H.P

c. 	CONVEYOR-1 ½ H.P

A N N E X U R E –X

TREATMENT  SCHEME OF POUCH PACKING

MINERAL WATER FROM MAIN MWP TANK

250 LTRS OVER HEAD SS TANK

20 LTRS TOP MOUNTED SS TANK VERTICAL SEALING

BOTTOM HORIZONTEL SEALING FILLING BY NOZZEL

UPPER HORIZONTEL SEALING

READY POUCH TO FILLING CONTAINER

A N N E X U R E – XI

TECHNICAL DATAS OF POUCH PACKING M/C

DESCRIPATION:

Mechanically 	operated 	Automatic 	form 	fill 	and 	Seal 	Machine 	for

packing 	Free 	Flow 	Liquids 	lime 	Mineral 	Water, 	in 	continues preprinted virgin film up to 100 ml to 250 ml.

PRODUCT 	: MINERAL WATER

CAPACITY 	: 2200 POUCHES/HR. (AVERAGE) DIMENSION 	: 2½’ X 3½’ X 6’ (Ft.) ( L X W X H) POWER 	: SINGLE PHASE-1 H.P.

A N N E X U R E – XII

SCHEDULE OF EXCLUSION

•	BACK LODING MACHINERY

•	POUCH ROLLS

•	SINGLE PHASE OF 2.H.P

•	CIVIL WORK OF PLAT FORM OF (L X W X H) 5 X 5 X 1

A N N E X U R E – XIII

ISI LAB EQUIPMENTS

| S.NO. | DESCRIPTION | TECHNICAL SPECIFICATION |
| --- | --- | --- |
| 1 | Binocular Research Microscope | Standard Objectives Combinations 4x, 10x, 100x Oil immersion Observation headinclined at with electronic variable intensity control |
| 2 | Analytical Balance Single Pan | Having readability of 0.1 mg to 200 gms. Dhonba or K.Roy |
| 3 | U.V. Spectrophotometer Systonic | The U.V. Spectrophotometer Systonic with a Cm & 5 cm cell. |
| 4 | Water Bath Double Walled Completer | Made SS with glass wool insulation. Thermostatic max Temperature 110 degree cent. Six holes each of 75 mm dia. With concentric ring |
| 5 | Auto Clave vertical | Complete made up of S.S with pressure gauge, safety valve, . steam release valve, constant pressure maintained at 15 lbs Size 30 X 30 cms |
| 6 | P.H. Meter Digital | Complete with combined calomel electrode stand & . dust cover table top |
| 7 | Hot Air Over | Inner Chamber SS Size 14" x 14" X 14" made of SS and outer body powder coated finish. Inner door with glass panel for observation. Temperature variable above ambient and radability 0.1 degree cent. |
| 8 | Incubator Bacteriological with Digital Indicator &Controller | Inner Chamber size 14" X 14" X 14" made of SS & outer body powder coated finish. Inner door with glass panel for observation. Temperature variable above ambientt and radability 0.1 degree cent. |
| 9 | Incubator Bacteriological with Digital Indicator &Controller | For 22 Degree Centrigrade |
| 10 | Incubator Bacteriological with Digital indicator & Controller | For 25 Degree centigrade |
| 11 | Bunsen Burner with Sopcock |  |
| 12 | Magnetic Shine with hot plate | Capaccity 2 Liter, Model 2 MOH |
| 13 | Colont Counter Digital with Audio Pen |  |
| 14 | SS Filter holder for micro filtration | 47 MM Dia |
| 15 | Laboratory high pressure Vaccum Pump | Diaphraragm type, Oil Free, 30 PSI Air Pressue & 20 Litewr/ Min. Air displacement. |
| 16 | Nephele Turbidity Meter Digital | With NTU Readability |
| 17 | Membrane Filters | 0.45 Microns, 47 MM Plain While " Satorius" |
| 18 | Inoculation Cahamber | With U.V. Lamp & Plain bulb Equitron |
| 19 | Hot Plate | With 20 cm dia energy, regulator and powder coated finish. |
| 20 | Glass Distillation Apparatus | Capacity - 1 Liter with heating mental |
| 21 | Gas Straipping Apparatus | One Set |
| 22 | Centrifuge | With Four Place 5000 RPM Capacity. REMI |
| 23 | Conductivity Meter |  |
| 24 | Platinum Crucible | 10 ml Approx, 10 grams wwith purity certificate |
| 25 | Furnac (Bunsen) |  |

A N N E X U R E – XIV

LIST OF CHEMICALS

| No. | Name of Chemical | Quantity |  |
| --- | --- | --- | --- |
| 1) | Potassium Chloroplatinate | 1 gm |  |
| 2) | Sodium Chloride | 500 gm |  |
| 3) | Hexamine | 500 gm |  |
| 4) | Hydrazine Sulphate | 100 gm |  |
| 5) | Buffer table : 7, 4, 9.2 | 1 Pkt. Each |  |
| 6) | Sulphuric Acid | 500 ml |  |
| 7) | Bromocresol Green | 125 ml |  |
| 8) | Glycerol | 500 ml |  |
| 9) | Hydrochloric Acid | 500 ml |  |
| 10) | Isopropyl Alcohol | 500 ml; |  |
| 11) | Sodium Sulphate | 100 gm |  |
| 12) | Glacial Acetic Acid | 500 ml |  |
| 13) | Potassium Iodide | 100 gm |  |
| 14) | Magnesium Sulphate | 500 gm |  |
| 15) | Potassium Dichromate | 500 gm |  |
| 16) | Starch | 500 gm |  |
| 17) | Ferric Chloride | 500 gm |  |
| 18) | Diammonium Hydrogen Phosphate | 500 gm |  |
| 19) | Potassium Chromate | 100 gm |  |
| 20) | Silver Nitrate | 5 gm |  |
| 21) | Potassium Nitrate | 500 gm |  |
| 22) | Sodium Sulphite | 500 gm |  |
| 23) | Antimony Metal Powder | 500 gm |  |
| 24) | Chromotropic Acid | 100 gm |  |
| 25) | Sulphanilamide | 100 gm |  |
| 26) | Sodium Nitrite | 500 gm |  |
| 27) | Trichlato Trifluoethane | 100 ml |  |
| 28) | Sodium Hydroxide | 500 gm |  |
| 29) | Azomethine –H- Sod. Salt | 1 gm |  |
| 30) | Murexide Indicator | 5 gm |  |

| 31) | EDTA | 100 gm |  |
| --- | --- | --- | --- |
| 32) | Calcium Carbonate | 500 gm |  |
| 33) | Ammonium Hydroxide sp. Gr. 0.92 | 500 ml |  |
| 34) | Chloroform | 500 ml |  |
| 35) | Hydroxylamine Hydrochloride | 100 gm |  |
| 36) | Neocuproin | 1 gm |  |
| 37) | Sodium Citrate | 500 gm |  |
| 38) | Copper Metal Powder | 500 gm |  |
| 39) | Eriochromide Black – T indicator | 25 gm |  |
| 40) | Methanol | 500 ml |  |
| 41) | Triethanolamine | 500 ml |  |
| 42) | Phenophthalin | 125 ml |  |
| 43) | Boric Acid | 500 gm |  |
| 44) | Zinc Sulphate | 500 gm |  |
| 45) | Ammonium Dichromate | 500 gm |  |
| 46) | Ammonium Acetate | 500 gm |  |
| 47) | Phosphoric Acid 85% | 500 gm |  |
| 48) | Rhodamine B | 25 gm |  |
| 49) | Perchloric Acid | 500 ml |  |
| 50) | Benzene | 500 ml |  |
| 51) | Formaldehyde | 500 ml |  |
| 52) | Citric Acid | 500 gm |  |
| 53) | 4-Amino Anti Pyrine | 25 gm |  |
| 54) | Ethyl Acetate | 500 ml |  |
| 55) | Silica Gel | 500 gm |  |
| 56) | Gelatin Powder | 500 gm |  |
| 57) | Aluminum Potassium Sulphate | 500 gm |  |
| 58) | Bromine Water Saturated | 100 ml |  |
| 59) | Ammonium Thiocyanate | 500 gm |  |
| 60) | Solvent Ether | 500 ml |  |
| 61) | Amyl Alcohol | 500 ml |  |
| 62) | Ammonium Carbonate | 500 gm |  |
| 63) | Hematoxylin Stain | 125 ml |  |
| 64) | Potassium Bisuphate | 500 gm |  |
| 65) | Ferrous Ammonium Sulphate | 500 gm |  |

| 66) | Potassium Permanganate | 500 gm |  |
| --- | --- | --- | --- |
| 67) | Thioglycolic Acid 80% | 500 ml |  |
| 68) | Hydrogen Peroxide 30% | 500 ml |  |
| 69) | Nitric Acid | 500 ml |  |
| 70) | Potassium Periodate | 100 gm |  |
| 71) | Sulphur Dioxide Saturated Solution | 500 ml |  |
| 72) | Phenol | 500 gm |  |
| 73) | Potassium Bromite | 500 gm |  |
| 74) | Potassium Promide | 500 gm |  |
| 75) | Copper Sulphate | 500 gm |  |
| 76) | Macconkey Agar | 100 gm |  |
| 77) | Violet Red Bile Agar | 100 gm |  |
| 78) | DRCM Agar | 100 gm |  |
| 79) | Pseudomonas Isolation Agar | 100 gm |  |
| 80) | Nutrient Agar | 11 gm |  |
| 81) | Cobalt Chloride | 100 gm |  |
| 82) | Sodium Thiosulphate | 500 gm |  |
| 83) | Ammonium Solution SRL | 500 ml |  |
| 84) | Propane 2-ol(LR) | 500 ml |  |
| 85) | O-Phosphoric Acid | 500ml |  |
| 86) | Mangenous Sulphate | 500 gm |  |
| 87) | Zinc Metal Dust | 500 gm |  |
| 88) | Thri Ethanol amine (LR) | 500 ml |  |
| 89) | Tergitol & Agar Medium | 100 gm |  |
| 90) | Yeast Extract Agar | 50 gm |  |
| 91) | Milk Agar | 100 gm |  |
| 92) | Yeast Extract Nrptj | 50 gm |  |
| 93) | Sodium Arsenite | 5 gm |  |
| 94) | Ammonium Oxalate | 500 gm |  |
| 95) | Ascorbic Acid | 100 gm |  |
| 96) | Charcoal Activated | 500 gm |  |
| 97) | Cadmium Sulphate | 500 gm |  |
| 98) | N-1 Nepthyal Ethelene Di Amine | 5 gm |  |
| 99) | N, N Di Methyl Pera Phenyls Di Amine Oxalic Acid | 5 gm |  |
| 100) | Lead Acetate | 500 gm |  |
| 101) | Do Decyl Benzene Sulphonic Methyl Ester | 5 gm |  |
| 102) | Erichrome Cynine | 25 gm |  |
| 103) | 1-10 Phenanthroline | 5 gm |  |
| 104) | Carbon Tetra Chloride | 100 |  |
| 105) | Calcium Hydroxidie | 100 gm |  |
| 106) | Chlor hexane | 100 ml |  |
| 107) | Ferric Citrate | 500 gm |  |
| 108) | 2-Hydroxy 1-3 Nephtholic acid (Patton’s & Reeders reagent) | 100 ml |  |
| 109) | Methyl orange | 5 gm |  |
| 110) | Nessler’s reagent | 100 gm |  |
| 111) | Potasium Chloride | 500 gm |  |
| 112) | Patassium Dihydrogen orhto phoshphate | 500 gm |  |
| 113) | Potassium hydroxide | 500 gm |  |
| 114) | Di-potassium hydrogen orhto phoshphate | 500 gm |  |

| 115) | Potassium hydrogen carbonate | 550 gm |  |
| --- | --- | --- | --- |
| 116) | Sodium Acetate | 500 gm |  |
| 117) | Sodium Carbonate | 500 gm |  |
| 118) | Sodium Sulphite | 500 gm |  |
| 119) | Universal pH indicator | 500 ml |  |
| 120) | Potasium Cyanide | 100 gm |  |
| 121) | Chlorhaxanone | 100 ml |  |
| 122) | Barium Chloride | 500 gm |  |

A N N E X U R E – XV

| LIST OF GLASSWARE S.NO. | DESCRIPTION | NOS |
| --- | --- | --- |
| 1 | Nessler Cylinder Capacity 50 ml | 6 |
| 2 | Glass Stoppered Bottled Capacity - 1 Liter | 20 |
| 3 | Desiccator Capacity-150 | 1 |
| 4 | Volumetric Flask Capacity 50 ml | 1 |
| 5 | Volumetric Flask Capacity 100 ml | 1 |
| 6 | Volumetric Flask Capacity 250 ml | 1 |
| 7 | Volumetric Flask Capacity 500 ml | 1 |
| 8 | Graduated Pipettes Capacity 0.1 ml Borosil | 1 |
| 9 | Graduated Pipettes Capacity 1 ml Borosil | 1 |
| 10 | Graduated Pipettes Capacity 10 ml Borosil | 1 |
| 11 | Graduated Pipettes Capacity 25 ml Borosil | 1 |
| 12 | Seperating Funnel Capacity 500 ml Borosil | 2 |
| 13 | Measuring Cylinder Capacity 50 ml Borosil | 1 |
| 14 | Measuring Cylinder Capacity 100 ml Borosil | 1 |
| 15 | Conical Flask Capacity 250 ml Borosil | 2 |
| 16 | Conical Flask Capacity 500 ml Borosil | 2 |
| 17 | Burette Capacity 50 ml Borosil | 1 |
| 18 | Erlenemeyer Flask Capacity 500 m l | 1 |
| 19 | Petridishes Size 4" | 6 |
| 20 | Test Tube iso/16 mm | 20 |
| 21 | Durshams Tubes 35/8 mm | 1 Pak. |
| 22 | Whatman Filter Paper No.40/41/42 size 110 mm | 1 Pak. |
| 23 | Beaker Capacity 100 ml Borosil | 5 |
| 24 | Wide month Bottle 1 Ltr with Glass Work | 1 |
| 25 | Suction Flask 500 ml | 1 |
| 26 | Rubber Tubing | 5 Mtrs |
| 27 | Burette Stand | 1 |
| 28 | Pipette Stand | 1 |

A N N E X U R E – XVI

SCHEDULE OF EXCLUSION

•	CIVIL WORKS OF PLAT FORM

•	WASH BASIN & WATER CONNETION FOR MINERAL WATER & SINGLE WATER

•	SIZE 2.5 EET HEIHT 2.25 FEET WIDTH

•	STAND FOR BOTTLES

•	1/05 A/C. BIOLOGICAL LAB

•	GAS CYLINDER

•	WOODEN/ACREYIL CHAMBERS FOR GAS STRIPPER

A N N E X U R E – XVIII

TERMS AND CONDITION

TERMS AND CONDITION:

•	PRICES ARE EX. FACTORY DELHI/GHAZIABAD.

•	VAT EXTRA AT ACTUAL AT THE TIME OF DELIVERY.

•	FORWARDING, TRANSPORTATION, OCTROI & INSURANCE SHALL BE EXTRA AT ACTUAL AT THE TIME OF DELIVERY.

•	ERRECTION & COMMI : INCLUSIVE

- 	To & Fro, + Lodging & Boding,+ Local Conveyance charges to buyers account.

- 	Necessary Tools, tackles, labor & Chemical should be provided by client.

•	PAYMENT TERMS :

- 40% ADVANCE ALONGWITH 	TECHNO – COMMERCIAL CLEAR ORDER 	&

- 60% BALANCE AGAINST PROFO. INVOICE, BEFORE DESPATCH.

•	DELIVERY :

- WITHIN 3 - 4 WEEKS AFTER RECEIPT OF YOUR ORDER 	WITH ADVANCE.

•	GUARANTEE / WARRANTY :

- WE CAN OFFER YOU GUARANTEE FOR ;

- 12 MONTHS FROM THE DATE OF SUPPLY OF THE PLANT

(NB : Guarantee clause is valid for Mfg. Defect/Workmanship Defect only

Our liability is limited to repair or replace of 	the 	same.)

A N N E X U R E – XIX

WHY ? MINERAL WATER PROJECT

HELP LINE

GENERAL HELP

1. To Provide  the Information  About the Profitability  of the Project  in

General.

2. Inform You About the Demand Existing in Your Area.

3. Site Selection considering Following Parameters :- a) Raw Water Quality

b) Space Availability

c) Location of Area

4. Free Raw Water Testing at Our Advanced Laboratory.

5. Designing Appropriate Scheme Duly Confirmed by Our Foreign Associates with

ISOMATRIC Drawing, G. A. Drawing, Photograph etc.

6. On Order Confirmation We will Submit the Following :-

* Total Building Construction Layout (If new Building is Consisting of all Hygienic Standards as per WHO, EEC, US FDA, ISI010500 Standards is to be Built.

* Offering Necessary Changes in Existing Building

(If Existing Building is to be Used).

MARKETING HELP

1. Recommending the Names of NID GUYS for Selection of Brand Name.

2. We can give You the Ideas and Help You in Preparation of Literatures, Action Plants for Marketing the Product (I.e. Recommending Marketing Team, etc.)

3. We can Help You in Executing the Formalities required to be done for the

Govt.

MINERAL	WATER	PROJECT

CAPACITY : 20,000 LITER/DAY (ISI APPROVED PLANT)

A: CAPITAL COST :

SHED & BUILDING 	10.00 LACS

| MINERAL WATER | PLANT | 07.00 | LACS |
| --- | --- | --- | --- |
| POUCH PACKING | MACHINE | 01.35 | LACS |
| ISI LAB |  | 04.25 | LACS |

| ISI FEE | & EXTRA | 01.10 | LACS |
| --- | --- | --- | --- |
| WORKING | CAP | 01.00 | LACS |

TOTAL PROJECT COST 	23.70 LACS

B: INTEREST COST : 	11%

| INTEREST | OF | THE 12 MONTHS | 2,60,700/- |
| --- | --- | --- | --- |
| INTEREST | OF | THE 1 MONTH | 21,725/- |
| INTEREST | OF | 1 DAYS | 724/- |

INTEREST (PER LITER) 	0.03/-

COST OF PRODUCTION

FOR BOTTLE :- 1 LTR

COST INCURRED FOR BOTTLE OF 1 LTR.

| A. | COST | OF | BOTTLE + CAP | Rs. | 2.90 |
| --- | --- | --- | --- | --- | --- |
| B. | COST | OF | LABEL SHRINK | Rs. | 0.25 |
| C. | COST | OF | CAP SHRINK | Rs. | 0.05 |
| D. | COST | OF | RO WATER | Rs. | 0.03 |
| E. | COST | OF | MISC | Rs. | 0.10 |

F. COST OF INTEREST PER LITER

|  | ON CAPITAL INVESTMENT |  |  | Rs. | 0.03 |
| --- | --- | --- | --- | --- | --- |
| G. | PACKING (CARTOON) COST | PER | BAG | Rs. | 0.50 |

TOTAL COST 	Rs. 3.86

FOR POUCHES :- 250 ml

| COST INCURRED IN MFG. 1 POUCH OF 250 ml WATER. A. COST | OF | EMPTY POUCH WITH | PRINTING | Rs. | 0.14 |
| --- | --- | --- | --- | --- | --- |
| B. COST | OF | WATER |  | Rs. | 0.02 |
| C. COST | OF | TRANSPORTATION |  | Rs. | 0.02 |

| D. MISCELLANEOUS COST | Rs. | 0.02 |
| --- | --- | --- |
| E. COST OF INTEREST PER LITER |  |  |
| ON CAPITAL INVESTMENT | Rs. | 0.03 |

TOTAL COST 	Rs. 0.23

FOR CARBOYS :- 20 LTRS

1 LTR WATER

| TOTAL COST (Per Ltr) Rs. 0.18 | A. COST | OF | WATER |  | Rs. | 0.03 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B. COST | OF | MAINTENANCE |  | Rs. | 0.05 |  |
|  | C. COST | OF | TRANSPORTATION |  | Rs. | 0.05 |  |
|  | D. COST | OF | INTEREST PER LITER |  |  |  |  |
| ON CAPITAL INVESTMENT |  |  |  | Rs. |  | 0.03 |  |
| E. MISC. EXP. (SALARY etc.) |  |  |  | Rs. |  | 0.02 |  |

TOTAL COST (Per JAR) 	Rs. 3.60

PROFITABILITY CHART

| PROFITIBILITY CHART | BOTTLE | POUCH | CARBOYS |
| --- | --- | --- | --- |
| SELLING COST | 6.00 | 0.30 | 20.00 |
| OPERATING COST | 3.86 | 0.23 | 3.60 |
| PROFIT PER ITEM | 2.14 | 0.07 | 16.40 |

| TOTAL PROFIT FOR WATER QTY. 20,000 LTRS : |  |  |  |
| --- | --- | --- | --- |
| ITEM WISE PACKING | LITERS | ITEM | PROFIT RS. |
| BOTTLE PACKING | 3000 | 3000 BOTTLES (2.14 X 3000) | 6,420 |
| POUCH PACKING | 13,000 | 52000 POUCH (0.07 X 52000) | 3,640 |
| JAR PACKING 20 LTRS) | 4,000 | 200 JAR (16.40 X 200) | 3,280 |
| TOTAL PROFIT PER DAY === TOTAL PROFIT PER MONTH === |  |  | 13,340 400,200 |

PAY BACK PERIOD OF THE PLANT	==   >	THREE MONTHS

A N N E X U R E –XX

SPECIAL 	FEATURES 	OF 	ASH R.O. PLANT

01. Plant design on latest state of art – european technology,compact,

decent.

02. We offered total imported systems ;

• Sealed Packed IS0-9001 MEMBRANE– HYDRAUNATICS – California, U.S.A.

• Micron Cartridge Filter Bag – AmeteK – Suziki – Japan

• Anthrecite

03. We offer our services for ;

• Designing of the plant,

• Prompt after Sales Services

04. We offer you different manufacturing stage wise inspection at our factory.

•	After fabrication of the unit,

•	At the time of assembling of the unit.

•	We can offer of you inspection while Fitting of sealed pack

Membrane.

•	We offer inspection of fully installed and running plant for 3

days at our factory with testing.

05. 	We 	are 	very 	close 	to 	you 	we 	offer 	best 	services. 	Besides 	our Technical Experts & Design Experts will visit frequently to your site for better performance.

06. Also visit of our Foreign Associates –if required.

07. SITPL, can offer Membrane Cleaning facility at its Factory, for enhancing life of Membrane Systems.

08. Plant is designed on 10% Higher capacity & T.D.S. i.e. 2795 ppm, for better result in future.

09. We can offer you 70% recovery of plant

10. MAINTENANCE 	FREE 	SYSTEMS.